- Please Un-compress the SecureModeUpgrade.rar. There will be five files in the uncompressed folder as below screenshot. Running tool of StartHere.exe

中文翻译：点击starthere.exe

- Please modify the IP of the PC to 192.168.1.200

中文翻译：修改电脑IP地址为192.168.1.200，如下图

- Connect PC and the phone use one cable.

中文翻译：用一个网线连接电脑网口和话机LAN口

- Please Keep pressing ‘*’ and ‘1’ keys and start up the phone until the LCD displays Upgrading information;

中文翻译：插上电源，长按‘*’和‘2’按键，看到tftpd日志上有加载升级日志

the phone will download the Kernel from the PC. Wait for about five minutes. After the download is done, the phone will upgrade automatically.

中文翻译：等待5分钟，话机会自动重启