- After Un-compress the SecureModeUpgrade.rar. There will be five files in the uncompressed folder as below screenshot. Running tool of StartHere.exe

- Please modify the IP of the PC to 192.168.1.200

- Connect PC and the phone use one cable.

- Please Keep pressing ‘*’ and ‘1’ keys and start up the phone until the LCD displays Upgrading information;

- the phone will download the Kernel from the PC. Wait for about five minutes. After the download is done, the phone will upgrade automatically.

Note: after upgrade the files, sometimes the phone would auto restart about 5 times, please also wait for it.

if the phone also cannot boot up, please replug the power and try to press the ‘*’ ‘1’ and ‘2’ three keys at same time, then the phone would update the bootloader.img.

after update the bootloader.img, if also cannot recover, try to update kernel again.